Bluetooth controlled car using mobile application

Introduction

In this project,The android app sends data packets to the Bluetooth module. The Bluetooth modules sends this data packet to Arduino Uno through Serial Communication. A Bluetooth-controlled car is a simple car that runs on a wireless Bluetooth signal. In Arduino, we have mainly  Bluetooth Options HC-06 module

Components

- Arduino nano

- Micrometal gear motor-600RPM 12v

- DRV8833 motor driver

- HC-06 bluetooth module

- Battery and battery holder

- Switch

- Connecting wires

- breadboard

Application

- The Bluetooth is used in wireless head sets.

- Bluetooth is used to transfer files, images and MP3 or MP4 between cell phones.

- It is used in laptops and notebooks.

- It is used in PDAs (personal digital assistant).

- It is also used in printers.

- It is almost used in wireless communication (WAN).

- It is used in wireless mouse and keyboards.

- It is used in data logging equipment data logging equipment that transmit data to a computer via Bluetooth technology.

- It is used for sending small advertisements from Bluetooth enabled advertising hoardings to other, discoverable, Bluetooth devices.

- It is used in short range transmission of health sensor data from medical devices to mobile phone,set top box or dedicated tele health

Objective

flowchart

Program

#include <SoftwareSerial.h>

SoftwareSerial BT(10, 11);

// creates a "virtual" serial port/UART

// connect BT module TX to D10

// connect BT module RX to D11

// connect BT Vcc to 5V, GND to GND

//we define the pins that set the motors' direction

int motor1a = 2;

int motor1b = 3;

int motor2a = 4;

int motor2b = 5;

void setup()

{

// we define the motor pins as outputs

pinMode(motor1a, OUTPUT);

pinMode(motor1b, OUTPUT);

pinMode(motor2a, OUTPUT);

pinMode(motor2b, OUTPUT);

BT.begin(9600);// set the data rate for the SoftwareSerial port

}

char a; // stores incoming character from other device

void loop()

{

if (BT.available())

// if text arrived in from BT serial

{

a=(BT.read());//it will be read and

if (a=='1')//if a=1 (the signal from the 'up' button from the bluetooth app) it will move forward

{

digitalWrite(motor1a, LOW);

digitalWrite(motor1b, HIGH);

digitalWrite(motor2a, LOW);

digitalWrite(motor2b, HIGH);

}

if (a=='2')//it will move backwards

{

digitalWrite(motor1a, HIGH);

digitalWrite(motor1b, LOW);

digitalWrite(motor2a, HIGH);

digitalWrite(motor2b, LOW);

}

if (a=='3')// it will move left

{

digitalWrite(motor1a, HIGH);

digitalWrite(motor1b, LOW);

digitalWrite(motor2a, LOW);

digitalWrite(motor2b, HIGH);

}

if (a=='4')//it will move right

{

digitalWrite(motor1a, LOW);

digitalWrite(motor1b, HIGH);

digitalWrite(motor2a, HIGH);

digitalWrite(motor2b, LOW);

}

if (a=='0')//it will not move

{

digitalWrite(motor1a, LOW);

digitalWrite(motor1b, LOW);

digitalWrite(motor2a, LOW);

digitalWrite(motor2b, LOW);

}

}

}

Hardware

- Connect the VCC of the HC-06 and DRV8833 to 5V

- Connect the GND of the HC-06 and DRV8833 to GND

- Connect the TXD to D10

- Connect the RXD to D11

- Connect INT1, INT2, INT3, INT4 to D2, D3, D4, D5 accordingly

- Connect the first motor's cables to OUT1 and OUT2

- Connect the second motor's cables to OUT3 and OUT4

- Connect the battery's "" to 5V and the "-" to GND